Digital Pirani Vacuum Gauge  DPRG-2GH

The UVS Model DPRG series Vacuum Pirani Gauge Controller displays vacuum pressure as measured from a S-PR-GH sensor tube. It displays vacuum measurements based on thermal conductivity of gases. The Model DPRG-2GH unit covers the range from 0.001mbar to 20 mbar, and controls relay with independent set point.

TECHNICAL SPECIFICATIONS

| Number of sensor channels | TWO |
| --- | --- |
| Display Range | 1000 mBar to 0.001mBar. |
| Measuring Range | 20 mBar to 0.001mBar. |
| Units of Display | m.Bar |
| Vacuum Gauge Head | S-PR-GH sensor tube |
| Channel selection | Through selection switch provided on the front panel |
| Temp. Range | +2 to +50 deg. Celsius |
| VAC. Display | Seven segment bright red LED, mantissa and exponential |
| VAC and ATM Adjust | adjusted by trim pot at the signal conditioner PCB. |
| Operating Voltage | universal power supply, input voltages from 90 ~ 264VAC 50/60 Hz |
| Mounting | ¼ DIN 92 x 92 mm (3.62” x 3.62") |
| Environmental Considerations | not for use with explosive or corrosive gases |
| Overall dimensions | 92mm X 92mm X 110mm Depth |
| Optional outputs on request | RS485, 0 -10V Dc analog output |

Features at a Glance

■ Measurement range 0.001 to 20 mbar ■ Space saving rugged design- 1/8 DIN console ■ Space saving rugged design- 1/8 DIN console ■ Mounts in any orientation ■ brass body measuring cell with metal-sealed feed through ■ with universal power supply – input voltage from 90 ~ 264VAC, frequency 47 ~ 440Hz

Applications

■ Controlling high vacuum systems ■ Fore vacuum pressure monitoring ■ Safety circuits in vacuum systems ■ General vacuum measurement & control in the fine and rough vacuum range.

Calibration:

Unique Vacuum Solutions offers a convenient recalibration service to help users of vacuum measurement pirani/ thermocouple/ full range gauges to maintain all of their vacuum gauges in Certified and peak condition, regardless of the original manufacturer. All makes of vacuum gauges can also be calibrated / repaired if required.

The calibration certificates are available only on request at additional cost. Pl contact before ordering.

UNIQUE VACUUM SOLUTIONS

#57, 8th cross, 1st B cross, Daddanna industrial area main road, vishwaneedam post, Near Peenya 2nd stage, Bangalore -560091

Ph: 080-28367059, 9886726920. e mail: support@uniquevacuum.co.in, WEB: www.uniquevacuum.co.in